# 📊 Arduino Data Logger (DS18B20 + Light Sensor + SD Card)

## 📌 Project Overview

This project implements a sensor data logger using an Arduino Uno. The system collects:

- 🌡 Temperature data from a DS18B20 digital sensor

- 💡 Light intensity data from a photoresistor

- 🕒 Timestamp data from an optional DS3231 RTC module

All readings are saved as timestamped CSV entries to an SD card. The system records at least 100 samples automatically.

## 1. Hardware (Wokwi / Arduino)

Components:

| Component | Pin / Connection |
| --- | --- |
| Arduino Uno | — |
| DS18B20 | D2 (data) + 4.7kΩ pull-up to 5V |
| Photoresistor | A0 (analog input) |
| SD card module | CS=D10, MOSI=D11, MISO=D12, SCK=D13 |
| RTC DS3231 (optional) | SDA=A4, SCL=A5 |

## 🧰 Hardware Components

- Arduino Uno

- DS18B20 Temperature Sensor

- Photoresistor (LDR)

- SD Card Module (SPI interface)

- DS3231 RTC Module (optional)

- 4.7kΩ resistor (pull-up for DS18B20)

### 🔌 Pin Connections

| Component | Arduino Pin |
| --- | --- |
| DS18B20 Data | D2 |
| Light Sensor | A0 |
| SD Card CS | D10 |
| SD MOSI | D11 |
| SD MISO | D12 |
| SD SCK | D13 |
| RTC SDA | A4 |
| RTC SCL | A5 |

## 📂 Output File (CSV Format)

File name: data.csv

Format:

Timestamp,Temperature_C,Light

2026-02-19 12:00:01,23.6,512

2026-02-19 12:00:02,23.7,518

...

- Minimum of 100 samples

- Comma-separated format

- Compatible with Excel, MATLAB, Python, etc.

## ⏱ Sampling Rate

- Sampling interval: 1 sample per second (1000 ms)

- Total time to collect 100 samples: ~100 seconds

- Controlled using:

const unsigned long sampleInterval = 1000;

### Why 1 second?

- Temperature and ambient light change slowly.

- Prevents unnecessary SD card writes.

- Balances data resolution and power efficiency.

## 🔋 Power Considerations

### 1️⃣   SD Card Power Usage

- Highest power consumer (~50–100 mA during write).

- Optimization strategy:

  - Buffer multiple readings in RAM before writing.

  - Reduce write frequency.

### 2️⃣    Arduino Power Optimization

- Use sleep mode between samples.

- Example (LowPower library):

- LowPower.powerDown(SLEEP_1S, ADC_OFF, BOD_OFF);

### 3️⃣   DS18B20 Efficiency

- Low standby current (~1 mA).

- Can operate in parasite power mode.

### 4️⃣   Sampling Trade-Off

Lower sampling rate → lower power consumption → longer battery life.

## ✅ Project Features

- Digital temperature sensing (high accuracy ±0.5°C)

- Real-time timestamp logging

- Structured CSV output

- Stops automatically after 100 samples

- Expandable for more sensors

💻 Arduino Code

#include <SPI.h>

#include <SD.h>

#include <Wire.h>

#include <RTClib.h>

#include <OneWire.h>

#include <DallasTemperature.h>

// Pins

const int chipSelect = 10;

const int lightPin = A0;

const int oneWireBus = 2;

// Setup OneWire and DallasTemperature

OneWire oneWire(oneWireBus);

DallasTemperature sensors(&oneWire);

// RTC

RTC_DS3231 rtc; // optional

// Sampling settings

const unsigned long sampleInterval = 1000; // 1 sample per second

unsigned long lastSampleTime = 0;

// SD file

File dataFile;

int sampleCount = 0; // counter for 100 samples

void setup() {

Serial.begin(9600);

// SD card init

if (!SD.begin(chipSelect)) {

Serial.println("SD init failed!");

while (1);

}

Serial.println("SD initialized.");

// DS18B20 init

sensors.begin();

// RTC init

if (!rtc.begin()) {

Serial.println("RTC not found!");

}

// Create CSV file

dataFile = SD.open("data.csv", FILE_WRITE);

if (dataFile) {

dataFile.println("Timestamp,Temperature_C,Light");

dataFile.close();

}

}

void loop() {

unsigned long currentTime = millis();

if (currentTime - lastSampleTime >= sampleInterval && sampleCount < 100) {

lastSampleTime = currentTime;

sampleCount++;

// Read sensors

int lightVal = analogRead(lightPin);

sensors.requestTemperatures();

float temperatureC = sensors.getTempCByIndex(0);

// Timestamp

DateTime now = rtc.now();

String timestamp = String(now.year()) + "-" + String(now.month()) + "-" + String(now.day()) + " "

+ String(now.hour()) + ":" + String(now.minute()) + ":" + String(now.second());

// Write to SD

dataFile = SD.open("data.csv", FILE_WRITE);

if (dataFile) {

dataFile.print(timestamp);

dataFile.print(",");

dataFile.print(temperatureC);

dataFile.print(",");

dataFile.println(lightVal);

dataFile.close();

Serial.print("Sample ");

Serial.print(sampleCount);

Serial.println(" saved.");

Serial.println(timestamp);

Serial.println(temperatureC);

Serial.println(lightVal);

} else {

Serial.println("Error opening file");

}

}

// Optional: stop after 100 samples

if (sampleCount >= 100) {

Serial.println("Collected 100 samples. Stopping loop.");

while (1); // stop

}

}